##### S P E C I F I C A T I O N S

# Description     LCM

### Model name   GTG-2401283-FT6C0C

- HEBEI GEM-TECH ELECTRONICS CO., LTD.

Contents

- 1. BASIC SPECIFICATION

- 2. ABSOLUTE MAXIMUM RATINGS

3. ELECTRICAL & OPTICAL CHARACTERISTICS

- 4. TIMING CHARACTERISTICS

- 5. FUNCTION DESCRIPTION& INSTRUCTION SET

- 6. CHARACTER FONT TABLE

- 7. DIMENSIONAL OUTLINE

8. QUALITY	SPECIFICATION

9. RELIABILITY

10. HANDLING PRECAUTION

11. PRECAUTION FOR USE

- 1. BASIC SPECIFICATION

1-1 DISPLAY SPECIFICATIONS

.  DISPLAY MODE      :    FSTN-TRANSMISSIVE-NEGATIVE

.  DISPLAY FORMAT    :    240 X 128 DOTS

.  INPUT DATA         :    8-BITS PARALLEL DATA INPUT FROM A MPU

.  MULTIPLEXING      :    1/128  DUTY

.  VIEWING DIRECTION :     6 O’CLOCK

.  DRIVED IC          :    T6963C (1 CHIP ) , KS0086 (5 CHIPS)

.  BEZEL               :    0.6T

.  BACKLIGHT         :     CCFL

.  OTHERS             :

- 1-2. MECHANICAL SPECIFICATION

| ITEM | SPECIFICATIONS | UNIT | REMARK |
| --- | --- | --- | --- |
| DIMENSIONAL OUTLINE | 159.4(W)×101.0(H)×13.0MAX.(T) |  | *REFERENCE |
| VIEW AREA | 124.0(W)×70.0(H) | mm | DIMENSIONAL |
| EFFECTIVE V/AREA | 119.97(W)×63.97(H) |  | OUTLINE |
| NUMBER OF CHARACTERS | 240 DOTS ×128 DOTS | -- |  |
| DOT PITCH | 0.50(W)×0.50(H ) | mm |  |
| DOT SIZE | 0.47(W)×0.47(H) | mm |  |

- 1-3 BLOCK DIAGRAM

- 1-4 TERMINAL FUNCTIONS

| PIN NO | SYMBOL | LEVEL | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | Vss | - | GROUND |
| 2 | VDD | - | POWER SUPPLY FOR LOGIC |
| 3 | Vo | - | CONTRAST ADJUST |
| 4 | C/D | H/L | WRITE : H:-COMMAND WRITE, L-DATA WRITE READ：H-STATUS READ，L-DATA READ |
| 5 | /WR | L | COMMAND AND DATA WRITE SIGNAL |
| 6 | /RD | L | DATA AND STATUS READ SIGNAL |
| 7 - 14 | DB0 – DB7 | H/L | DATA BUS |
| 15 | /CE | L | CHIP ENABLE SIGNAL |
| 16 | /RST | L | RESET SIGNAL |
| 17 | VOUT | - | NEGATIVE VOLTAGE OUTPUT |
| 18 | /DISPOFF | L | DISPLAY OFF |
| 19 | NC | - | NO CONNECTION |
| 20 | NC | - | NO CONNECTION |

- 1-5 POWER SUPPLY CIRCUIT AND CONTRAST ADJUST

- 2. ABSOLUTE  MAXIMUM  RATINGS ( Ta=25oC, VSS=0V)

| PARAMETER | SYMBOL | RATINGS |  |  | UNITS |
| --- | --- | --- | --- | --- | --- |
|  |  | MIN. | TYP. | MAX. |  |
| POWER SUPPLY FOR LOGIC | VDD-Vss | 0 | - | 7.5 | V |
| POWER SUPPLY FOR LCD DRIVER | VDD～Vo | 0 | - | 19.5 | V |
| INPUT VOLTAGE | VIN | Vss | - | VDD | V |
| OPERATING TEMPERATURE | Topr | 0 | - | 50 | oC |
| STORAGE TEMPERATURE | Tstg | -20 | - | 60 | oC |

- 3.ELECTRICAL & OPTICAL CHARACTERISTICS

3-1 ELECTRICAL CHARACTERISTICS (Ta=25 oC)

| ITEM | SYMBOL | CONDITION | MIN | TYPE | MAX. | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| LOGIC CIRCUIT |  |  |  |  |  |  |
| POWER SUPPLY | VDD-Vss | ——— | 4.8 | 5.0 | 5.2 | V |
| VOLTAGE |  |  |  |  |  |  |
| INPUT VOLTAGE | VIH | ——— | 2.2 | — | VDD | V |
| INPUT VOLTAGE | VIL | ——— | Vss | — | 0.6 | V |
| LOGIC CIRCUIT |  |  |  |  |  |  |
| POWER SUPPLY | IDD | VDD-Vss=5.0V | —— | 15.0 | 25.0 | mA |
| CURRENT |  |  |  |  |  |  |
| RECOMMENDED | VDD-Vadj |  |  |  |  |  |
| LCD DRIVING | =0 | Ta=25 oC | —— | 15.6 | —— | V |
| VOLTAGE | =0 |  |  |  |  |  |
| FRAME FREQUNCY | fFLM | - | - | 60 | ----- | Hz |

3-2. ELECTRO—OPTICAL CHARACTERISTICS( Ta=25 oC VDD=5.00.25V VOP=15.6V)

| ITEM | SYMBOL | CONDITION | MIN | TYPE | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VIEW ANGLE | Δ | =0o,Cr2 -90 o <1, 2<90 o | 35 | 40 | — | Deg |
| CONTRAST | Cr | =0 o,=0 o | 4 | 10 | — | — |
| RESPONSE TIME | tr(rise) | =0 o,=0 o | — | 250 | 300 | ms |
|  | tf(fall) | =0 o,=0 o | — | 300 | 350 | ms |

NOTE1: Definition of Viewing Angle ,

NOTE2: Definition of viewing Angle Range: Δ=|2-1|

NOTE3: Definition of Contrast

NOTE4: Definition of Response Time

3-3.  CCFL BACK-LIGHT SPECIFICATION

ELECTRICAL/ OPTICAL SPECIFICATIONS

| ITEM | STANDARD VALUE |  |  | UNIT | CONDITIONS |
| --- | --- | --- | --- | --- | --- |
|  | MIN. | TYP. | MAX |  |  |
| OP.FREQUENCY | - | 40000 | - | Hz | TA=25 oC |
| START VOLTAGE | - | 375 | - | AC Vrms |  |
| OP.VOLTAGE | - | 250 | - | AC Vrms |  |
| LUMINOUS INTENSITY | - | 400 | - | cd/m2 |  |

- 4. TIMING CHARACTERISTICS

4-1. INTERFACE TIMING CHART

| ITEM | SYMBOL | CONDITION | MIN. | MAX. | UNIT |
| --- | --- | --- | --- | --- | --- |
| C/D SET UP TIME | tCDS | VDD=5.0V Vss=0V Ta=25 oC | 100 |  | ns |
| C/D HOLD TIME | tCDH |  | 10 |  |  |
| C/D, RD, WR PULSE WIDTH | tCE, tRD, tWR |  | 80 |  |  |
| DATA SET UP TIME | tDS |  | 80 |  |  |
| DATA HOLD TIME | tDH |  | 40 |  |  |
| ACCESS TIME | tACC |  | - | 150 |  |
| OUTPUT HOLD TIME | tOH |  | 10 | 50 |  |

4-2 CPU READ/WRITE OPERATION SEQUENCE

- 5. FUNCTION DESCRIPTION& INSTRUCTION SET

● In this module :  It is work in single-scan, the RAM size is 8K, the address from 0000H-1FFFH.

● Alter power on, it is necessary to reset. RESET is kept L between 5 clocks up (oscillation clock).

● When HALT=L, the oscillation stops. The power supply for the LCD must now be turned off, to protect the LCD from DC bias.

● The HALT function includes the RESET function.

● The column /line counter and display register are cleared by RESET. (Other registers are not cleared.) Disable the display using the clear-display register.

● The status must be checked before data or commands are sent. The MSB=0 status check must be done in particular. There is a possibility of erroneous operation due to a hard interrupt.

● STA0 and STA1 must be checked at the same time. When a command is executed, data transmission errors may occur.

● The T6963C can only handle one byte per machine cycle (16 clocks). It is impossible to send more than two data in a machine cycle.

● When using a command with operand data, it important to send the data first, and then execute the command.

● The character codes used by the T6963C are different from ASCII codes.

- State after RESET/HALT

| TERMIAL | HALT | RESET |
| --- | --- | --- |
| D0 to D7 | F | F |
| d0 to d7 | F | F |
| r/w | H | H |
| ce | H (Note1) | H (Note1) |
| ad0 to ad15 | H (Note2) | H (Note2) |
| ce0,ce1 | H (Note1) | H (Note1) |
| ED,HOD | Final data | Final data |
| HSCP | L | L |
| LP | L | L |
| CDATA | H | H |
| FR | H | H |
| CH1 | L | KO |
| CH2 | L | VEND |
| DSPON | L | L |
| XO | H | OSC clock |

H: level H

L: level L

F: Floating (high impedance)

K0: Test signal

VEND: Test signal

(Note 1): In Attribute mode, H or L according to state of graphic pointer

(Note 2): in Attribute mode, data of graphic pointer

● RAM Interface

The external RAM is used to store display data (text, graphic and external CG data).

With single-scan, text data, graphic data and external CG data can be freely allocated to the memory area ( 64KB max.)

With dual-scan, LCD Ⅰis allocteed to 0000H to 7FFFH (32 KB max.), LCDⅡis allocated to 8000H to FFFFH (32 KB max.). Text data, graphic data and external CG data can be freely allocated in LCDⅠ. In LCDⅡ,the same address must be allocated as in LCDⅠ, except ad15. ad15 determines selection of LCDⅠor LCDⅡ.

It can be use the address decoded signals ce0( 0000 to 07FFH),ce1(0800 to 0FFFH) within 4 KB.ce0 and ce1 allow decoding of addresses in the ranges (0000 to 07FFH) and (0800 to 0FFFH) respectively within a 4-KB memory space.

(Example)

(1) Single-Scan                 (2) Dual-scan

TEXT

AREA

GRAPHIC

AREA

CG RAM

AREA

8000H

8FFFH

F7FFH

FFFFH

TEXT

AREA

GRAPHIC

AREA

CG RAM

AREA

0000H

3FFFH

77FFH

7FFFH

0000H

TEXT

AREA

GRAPHIC

AREA

CG RAM

AREA

7FFFH

F7FFH

FFFFH

CG : Character Generator

● Flowchart of communications with MPU

- Status Read

A status check must be performed before data is read or written.

Status check

The Status of T6963C can be read from the data lines.

RD      L

WR     H

CE      L

C/D     H

D0 to D7  Status word

The T6963C status word format is as follows:

MSB                                                       LSB

| STA7 D7 | STA6 D6 | STA5 D5 | STA4 D4 | STA3 D3 | STA2 D2 | STA1 D1 | STA0 D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| STA0 | Check command execution capability | 0: Disable 1: Enable |
| --- | --- | --- |
| STA1 | Check data read/write capability | 0: Disable 1: Enable |
| STA2 | Check Auto mode data read capability | 0: Disable 1: Enable |
| STA3 | Check Auto mode data write capability | 0: Disable 1: Enable |
| STA4 | Not used |  |
| STA5 | Check controller operation capability | 0: Disable 1: Enable |
| STA6 | Error flag. Used for screen peek and screen copy commands. | 0: Disable 1: Enable |
| STA7 | Check the blink condition | 0: Disable 1: Enable |

(Note 1) it is necessary to check STA0 and STA1 at the same time.

There is a possibility of erroneous operation due to a hardware interrupt.

(Note 2) for most modes STA0/STA1 are used as a status check.

(Note 3) STA2 and STA3 are valid in Auto mode; STA0 and STA1 are invalid.

STATUS

AUTO MODE STATUS

Status checking flow

STA0=1

STA1=1

STA0=1

STA1=1

a)                                 b)

*1

STA2=1 (Read)

or

STA3=1 (Write)

YES

YES

NO

NO

RETURN

RETURN

(Note 4) When using the MSB=0 command, a status read must be performed.

If a status check is not carried out, the T6963C can not operate normally, even after a delay time.

The hardware interrupt occurs during the address calculation period (at the end of each line).

If a MSB=0 command is sent to the T6963C during this period, the T6963C enters wait status.

If a status check is not carried out in this state before the next command is sent, There is the possibility that the command or data will not be received.

- Setting data

When using the T6963C, first set the data, then set the command.

Procedure for sending a command

a) the case of 1 data                    b) the case of 2 data

Send Command

Status check

Data write

Status check

Command write

End

STA0,1

Send Command

Status check

Data write

Status check

Data write

Status check

Command write

END

STA0,1

(Note) When sending more than two data, the last datum (or last two data) is valid.

● COMMAND DEFINITIONS

| COMMAND | CODE | D1 | D2 | FUNCTION |
| --- | --- | --- | --- | --- |
| REGIDTERS SETTING | 00100001 00100010 00100100 | X address Data Low address | Y address 00H High address | Set Cursor Pointer Set Offset Register Set Address Pointer |
| SET CONTROL WORD | 01000000 01000001 01000010 01000011 | Low address Columns Low address Columns | High address 00H High address 00H | Set Text Home Address Set Text Area Set Graphic Home Address Set Graphic Area |
| MODE SET | 1000x000 1000x001 1000x011 1000x100 10000xxx 10001xxx | — — — — — — | — — — — — — | OR mode EXOR mode AND mode Text Attribute mode Internal CG ROM mode External CG RAM mode |
| DISPLAY MODE | 10010000 1001xx10 1001xx11 100101xx 100110xx 100111xx | — — — — — | — — — — — | Display off Cursor on, blink off Cursor on, blink on Text on, graphic off Text off, graphic on Text on, graphic on |
| CURSOR PATTERN SELECT | 10100000 10100001 10100010 10100011 10100100 10100101 10100110 10100111 | — — — — — — — | — — — — — — — | - line cursor - line cursor - line cursor - line cursor - line cursor - line cursor - line cursor - line cursor |
| DATA AUTO READ/WRITE | 10110000 10110001 10110010 | — — — | — — — | Set Data Auto Write Set Data Auto Read Auto Reset |
| DATA READ/WRITE | 11000000 11000001 11000010 11000011 11000100 11000101 | Data — Data — Data — | — — — — — — | Data Write and Increment ADP Data Read and Increment ADP Data Write and Decrement ADP Data Read and Decrement ADP Data Write and Nonvariable ADP Data Read and Nonvariable ADP |
| SCREEN PEEK | 1110000 | — | — | Screen Peek |
| SCREEN COPY | 11101000 |  |  | Screen Copy |
| BIT SET/RESET | 11110xxx 11111xxx 1111x000 1111x001 1111x010 1111x011 1111x100 1111x101 1111x110 1111x111 | — — — — — — — — — | — — — — — — — — — | Bit Reset Bit Set Bit 0 (LSB) Bit 1 Bit 2 Bit 3 Bit 4 Bit 5 Bit 6 Bit 7 (MSB) |

● Setting registers

| CODE | HEX. | FUNCTION | D1 | D2 |
| --- | --- | --- | --- | --- |
| 00100001 | 21H | SET CURSOR POINTER | X ADRS | Y ADRS |
| 00100010 | 22H | SET OFFSET REGISTER | DATA | 00H |
| 00100100 | 24H | SET ADDRESS POINTER | LOW ADRS | HIGH ADRS |

- Set Cursor Pointer

The position of the cursor is specified by X ADRS and Y ADRS. The cursor position can only be moved by this command. Data read/write from the MPU never change the cursor pointer. X ADRS and Y ADRS are specified as follows.

ADRS and Y ADRS are specified as follows.

X ADRS 00H to 4FH (lower 7 bits are valid)

Y ADRS 00H to 1FH (lower 5 bits are valid)

a) Single-Scan                           b) Dual-Scan

Y ARDS 00H to 0FH

Upper screen

Y ADRS 00H to 0FH

Y ADRS 10H to 1FH

Lower screen

(2) Set Offset Register

the offset register is used to determine the external character generator RAM area.

The T6963C has a 16-bit address bus as follows.

MSB                                                                      LSB

| ad15~ad11 | ad10~ad3 | ad2~ad0 |
| --- | --- | --- |

Offset Register Data                Character Code               Line Scan

T6963C assign External character generator, when character code set 80H to FFH in using internal character generator. Character code 00H to 80H assign External character generator, when External generator mode.

The senior five bits define the start address in external memory of the CG RAM area. The next eight bits represent the character code of the character. In internal CG ROM mode, character codes 00H to 7FH represent the predefined “internal” CG RAM mode, character codes 80H to FFH represent the user’s own “external” characters. In external CG RAM mode, all 256 codes from 00H to FFH can be used to represent the user’s own character. The three least significant bits indicate one of the eight rows of eight dots that define the character’s shape.

The relationship between display RAM address and offset register

Offset register data    CGRAM hex. Address (start to end)

Offset register data

00000

00001

00010

11100

11101

11110

11111

CG RAM hex.address (start to end)

0000 to 07FFH

0800 to 0FFFH

1000 to 17FFH

E000 to E7FFH

E800 to EFFFH

F000 to F7FFH

F800 to FFFFH

Example 1)

Offset register                        02H

Character code                        80H

Character generator RAM start address 0001  0100  0000  0000

1    4     0     0    H

(address)       (data)

1400H            00H

1401H            1FH

1402H            04H

1403H            04H

1404H            04H

1405H            04H

1406H            04H

1407H            00H

(Example) The relationship between display RAM data and display characters

AB

(RAM DATA)   (Character)

21H               A

22H               B

83H               

 are displayed by character generator RAM.

(3) Set Address Pointer

The Set Address Pointer command is used to indicate the start address for writing to (or reading from) external RAM.

The flowchart for set address pointer command

Set Address Pointer

Status check

Set Address data

( lower 8 bits)

Status check

Set Address data

(upper 8 bits)

Status check

Send command 24H

END

STA0,1

Send Set Address Pointer command

● Set Control Word

| CODE | HEX. | FUNCTION | D1 | D2 |
| --- | --- | --- | --- | --- |
| 01000000 | 40H | Set Text Home Address | Low address | High address |
| 01000001 | 41H | Set Text Area | Columns | 00H |
| 01000010 | 42H | Set Graphic Home Address | Low address | High address |
| 01000011 | 43H | Set Graphic Area | Columns | 00H |

The home address and column size are defined by  this command.

- Set Text Home Address

The starting address in the external display RAM for text display is defined by this command.

The text home address indicates the leftmost and uppermost position.

| TH |  | TH+CL |
| --- | --- | --- |
| TH+TA |  | TH+TA+CL |
| (TH+TA)+TA |  | TH+2TA+CL |
| (TH+2TA)+TA |  | TH+3TA+CL |
| TH+(n-1)TA |  | TH+(n-1)TA+CL |

The relationship between external display RAM address and display position.

TH: Text home address

TA: Text area number (columns)

CL: Columns are fixed by hardware (pin-programmable)

Example)

Text home address                 : 0000H

Text area                        :  0020H

MD2=H,MD3=H                  : 32 columns

DUAL=H, MDS=L,MD0=L,MD1=H  : 4 lines

| 0000H | 0001H |  | 001EH | 001FH |
| --- | --- | --- | --- | --- |
| 0020H | 0021H |  | 003EH | 003FH |
| 0040H | 0041H |  | 005EH | 005FH |
| 0060H | 0061H |  | 007EH | 007FH |

- Set Graphic Home Address

The starting address of the external display RAM used for graphic display is defined by this command. The graphic home address indicates the leftmost and uppermost position.

The relationship between external display RAM address and display position

| GH |  | GH+CL |
| --- | --- | --- |
| GH+GA |  | GH+GA+CL |
| (GH+GA)+GA |  | GH+2GA+CL |
| (GH+2GA)+GA |  | GH+3GA+CL |
| GH+(n-1)GA |  | GH+(n-1)GA+CL |

GH: Graphic home address

GA: Graphic area number (columns)

CL: Columns are fixed by hardware (pin-programmable)

Example)

Graphic home address                 : 0000H

Graphic area                        :  0020H

MD2=H,MD3=H                  : 32 columns

DUAL=H, MDS=L,MD0=H,MD1=H  : 2 lines

| 0000H | 0001H |  | 001EH | 001FH |
| --- | --- | --- | --- | --- |
| 0020H | 0021H |  | 003EH | 003FH |
| 0040H | 0041H |  | 005EH | 005FH |
| 0060H | 0061H |  | 007EH | 007FH |
| 0080H | 0081H |  | 009EH | 009FH |
| 00A0H | 00A1H |  | 00BEH | 00BFH |
| 00C0H | 00C1H |  | 00DEH | 00DFH |
| 00E0H | 00E1H |  | 00FEH | 00FFH |
| 0100H | 0101H |  | 011EH | 011FH |
| 0120H | 0121H |  | 013EH | 013FH |
| 0140H | 0141H |  | 015EH | 015FH |
| 0160H | 0161H |  | 017EH | 017FH |
| 0180H | 0181H |  | 019EH | 019FH |
| 01A0H | 01A1H |  | 01BEH | 01BFH |
| 01C0H | 01C1H |  | 01DEH | 01DFH |
| 01E0H | 01E1H |  | 01FEH | 01FFH |

(3) Set Text Area

The display columns are defined by the hardware setting. This command can be to adjust the column of the display.

(Example)

LCD size                        : 20 columns, 4 line

Text home address                 : 0000H

Text area                        :  0014H

MD2=H,MD3=H                  : 32 columns

DUAL=H, MDS=L,MD0=L,MD1=H  : 4 lines

| 0000 | 0001 | -------- | 0013 | 0014 | -------- | 001F |
| --- | --- | --- | --- | --- | --- | --- |
| 0014 | 0015 | -------- | 0027 | 0028 | -------- | 0033 |
| 0028 | 0029 | -------- | 003B | 003C | -------- | 0047 |
| 003C | 003D | -------- | 004F | 0050 | -------- | 005B |

LCD

(4) Set Graphic Area

The display columns are defined by the hardware setting. This command can be used to adjust the columns of the graphic display.

(Example)

LCD size                        : 20 columns, 2 line

Graphic home address              : 0000H

Graphic area                      :  0014H

MD2=H,MD3=H                  : 32 columns

DUAL=H, MDS=L,MD0=H,MD1=H  : 2 lines

| 0000 | 0001 | -------- | 0013 | 0014 | -------- | 001F |
| --- | --- | --- | --- | --- | --- | --- |
| 0014 | 0015 | -------- | 0027 | 0028 | -------- | 0033 |
| 0028 | 0029 | -------- | 003B | 003C | -------- | 0047 |
| 003C | 003D |  | 004F | 0050 |  | 005B |
| 0050 | 0051 |  | 0063 | 0064 |  | 006F |
| 0064 | 0065 |  | 0077 | 0078 |  | 0083 |
| 0078 | 0079 |  | 008B | 008C |  | 0097 |
| 008C | 008D |  | 009F | 00A0 |  | 00AB |
| 00A0 | 00A1 |  | 00B3 | 00B4 |  | 00BF |
| 00B4 | 00B5 |  | 00C7 | 00C8 |  | 00D3 |
| 00C8 | 00C9 |  | 00DB | 00DC |  | 00E7 |
| 00DC | 00DD |  | 00EF | 00F0 |  | 00FD |
| 00F0 | 00F1 |  | 0103 | 0104 |  | 011F |
| 0104 | 0105 |  | 0127 | 0128 |  | 0123 |
| 0128 | 0129 |  | 013B | 013C |  | 0147 |
| 013C | 013D |  | 014F | 0150 |  | 015B |

LCD

If the graphic area setting is set to match the desired number of columns on the LCD, the addressing scheme will be automatically modified so that the start address of each line equals the end address of the previous line +1.

● Mode set

| CODE | FUNCTION | OPERAND |
| --- | --- | --- |
| 1000X000 | OR Mode | ─ |
| 1000X001 | EXOR Mode | ─ |
| 1000X011 | AND Mode | ─ |
| 1000X100 | TEXT ATTRIBUTE Mode | ─ |
| 10000XXX | Internal Character Generator Mode | ─ |
| 10001XXX | External Character Generator Mode | ─ |

X: invalid

The display mode is defined by this command. The display mode does not change until the next command is sent. The logical OR, EXOR, AND of text or graphic display can be displayed.

In Internal Character Generator mode, character codes 00H to 7FH are assigned to the built-in character generator ROM. The character codes 80H to FFH are automatically assigned to the external character generator RAM.

(Example)

GRAPHIC

TEXT

“OR”

“AND”

“EXOR”

(Note) Attribute functions can only be applied to text display, since the attribute data is placed in the graphic RAM area.

Attribute functions

The attribute operations are Reverse display, Character blink and Inhibit. The attribute data is written into the graphic area which was defined by the Set Control Word command. Only text display is possible in Attribute Function mode; graphic display is automatically disabled. However, the Display Mode command must be used to turn both Text and Graphic on in order for the Attribute function to be available.

The attribute data for each character in the text area is written to the same address in the graphic area. The Attribute function is defined as follows.

Attribute RAM 1byte

| X | X | X | X | d3 | d2 | d1 | d0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| d3 | d2 | d1 | d0 | FUNCTION |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | Normal display |
| 0 | 1 | 0 | 1 | Reverse display |
| 0 | 0 | 1 | 1 | Inhibit display |
| 1 | 0 | 0 | 0 | Blink of normal display |
| 1 X: invalid | 1 | 0 | 1 | Blink of reverse display |
| 1 | 0 | 1 | 1 | Blink of inhibit display |

● Display mode

| CODE | FUNCTION | COERAND |
| --- | --- | --- |
| 10010000 | Display off | ─ |
| 1001xx10 | Cursor on, blink off | ─ |
| 1001xx11 | Cursor on, blink on | ─ |
| 100101xx | Text on, graphic off | ─ |
| 100110xx X: invalid | Text off, graphic on | ─ |
| 100111xx | Text on, Graphic on | ─ |

| Cursor blink on: 1, off: 0 Cursor display on: 1, off: 0 Text display on: 1, off: 0 Graphic display: 1, off: 0 1 | 0 | 0 | 1 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

(Note) It is necessary to turn on “Text display” and “Graphic display” in the following cases.

- Combination of text/graphic display

- Attribute function

● Cursor Pattern Select

| CODE | FUNCTION | OPERAND |
| --- | --- | --- |
| 10100000 | 1-line cursor | ─ |
| 10100001 | 2-line cursor | ─ |
| 10100010 | 3-line cursor | ─ |
| 10100011 | 4-line cursor | ─ |
| 10100100 | 5-line cursor | ─ |
| 10100101 | 6-line cursor | ─ |
| 10100110 | 7-line cursor | ─ |
| 10100111 | 8-line cursor | ─ |

When cursor display is ON, this command selects the cursor pattern in the range 1 line to 8 lines.

The cursor address is defined by the Cursor Pointer Set command.

1-line cursor

2-line cursor

8-line cursor

● Data Auto Read/Write

| CODE | HEX. | FUNCTION | OPERAND |
| --- | --- | --- | --- |
| 10110000 | B0H | Set Data Auto Write | ─ |
| 10110001 | B1H | Set Data Auto Read | ─ |
| 10110010 | B2H | Auto Reset | ─ |

This command is convenient for sending a full screen of data from the external display RAM. After setting Auto mode, a Data Write (or Read) command is need not be sent between each datum. A Data Auto Write (or Read) command must be sent after a Set Address Pointer command. After this command, the address pointer is automatically incremented by 1 after each datum. In Auto mode, the T6963C cannot accept any other commands.

The Auto Reset command must be sent to the T6963C after all data has been sent, to clear Auto mode.

(Note) A Status check for Auto mode

(STA2,STA3 should be checked between sending of each datum. Auto Reset should be performed after checking STA3=1 (STA2=1). Refer to the following flowchart.

Auto write start

Auto read start

a) Auto Read mode                    b) Auto Write mode

Set Address Pointer

Status check 1

Status check 2

Status check 2

Status check 2

Status check 2

Auto write B0H

Data write

Data write

Auto reset B2H

STA0,1

STA3

Repetition

Set Address Pointer

Status check 1

STA0,1

Auto read B1H

Status check 2

STA2

Data read

Status check 2

Repetition

Data read

Auto reset B2H

END

END

Set Address Pointer

Status check 1

Status check 1

Status check 1

Status check 1

STA0=1

STA1=1?

N0

Status check 2

STA2=1

(STA3=1)?

Set Address data

(Lower 8 bits)

Set address data

(Upper 8 bits)

Set Address Pointer

STA0,1

Send Set Address Pointer command

YES

N0

YES

RETURN

RETURN

END

● Data Read/Write

| CODE | HEX. | FUNCTION | OPERAND |
| --- | --- | --- | --- |
| 11000000 | C0H | Data Write and Increase ADP | Data |
| 11000001 | C1H | Data Read and Increase ADP | ─ |
| 11000010 | C2H | Data Write and Decrease ADP | Data |
| 11000011 | C3H | Data Read and Decrease ADP | ─ |
| 11000100 | C4H | Data Write and Nonvarible ADP | Data |
| 11000101 | C5H | Data Read and Nonvarible ADP | ─ |

This command is used to for writing data from the MPU to external display RAM, and reading data from external display RAM to the MPU. Data Write/Data Read should be executed after setting address using Set Address Pointer command. The address pointer can be automatically incremented or decremented using this command.

(Note) This command is necessary for each 1-byte datum.

Refer to the following flowchart.

Data write start

Set Address Pointer

Status check 1

Status check 1

Set write data

Data write C0H

STA0,1

(Example)

Address pointer=1000H

(Example)

Data=AAH

AAH is written in 1000H address

Address pointer is 1001H

END

● Screen Peek

| CODE | HEX. | FUNCTION | OPERAND |
| --- | --- | --- | --- |
| 11100000 | E0H | Screen Peek | ─ |

This command is used to transfer 1 byte of display data to the data stack; this byte can then be read from the MPU by data access. The logical combination of text and graphic display data on the LCD screen can be read by this command.

The status (STA6) should be checked just after Screen Peek command. If the address determined by the Set Address Pointer command is not in the graphic area, this command is ignored and a status flag (STA6) is set

Refer to the following flowchart.

Screen peek start

Set Address Pointer

Status check 1

(*)

Set peek E0H

STA0,1

(*) Status check STA6=0?

NO

YES

Status check 1

Data access

END

(Note) This command is available when hardware column number and software column number are the same. Hardware column number is related to MD2 and MD3 setting.

Software column number is related to Set Text Area and Set Graphic Area command.

● Screen copy

| CODE | HEX. | FUNCTION | OPERAND |
| --- | --- | --- | --- |
| 11101000 | E8H | Screen Copy | ─ |

This command copies a single raster line of data to the graphic area.

The start point must be set using the set address pointer command.

(Note 1) If the attribute function is being used, this command is not available.

(With Attribute data is graphic area data.)

(Note  2) With Dual-scan, this command cannot be used (because the T6963C cannot separate the upper screen data lower screen data).

Refer to the following flowchart.

Screen copy start

Set Address Pointer

Status check 1

(*1)

(*2)

Screen copy E8H

STA0,1

NO

(*1) Status check STA6=1

(*2) Status check STA0/1=1

NO

YES

END

(Note) This command is available when hardware column number and software column number are the same. Hardware column number is related to MD2 and MD3 setting. Software column number to Set Text Area and Set Graphic Area command.

● Bit set/reset

| CODE | FUNCTION | OPERAND |
| --- | --- | --- |
| 11110xxx | Bit Reset | ─ |
| 11111xxx | Bit Set | ─ |
| 1111x001 | Bit 0 (LSB) | ─ |
| 1111x001 | Bit 1 | ─ |
| 1111x010 | Bit 2 | ─ |
| 1111x011 | Bit 3 | ─ |
| 1111x100 | Bit 4 | ─ |
| 1111x101 X: invalid | Bit 5 | ─ |
| 1111x110 | Bit 6 | ─ |
| 1111x111 | Bit 7 (MSB) | ─ |

This command use to set or reset a bit of the byte specified by the address pointer.

Only one bit can be set/reset at a time.

Refer to the following flowchart.

Bit Set/Reset

Set Address Pointer

Status check 1

Set write data

Status check 1

Bit set (reset)

END

STA0,1

- 6. CHARACTER FONT TABLE

[ CORRESPONDENCE BETWEEN CHARACTER CODE AND CHARACTER PATTERN]

- 7. DIMENSIONAL OUTLINE

8.QUALITY	SPECIFICATION

8-1.ACCEPTABLE QUALITY LEVEL

| Inspection items | Sampling procedures | AQL |
| --- | --- | --- |
| Visual-operating (Electro-optical) | MIL-STD-105D Inspection level Ⅰ Normal inspection Single sample inspection | 1.5 |
| Visual-not operating | MIL-STD-105D Inspection level Ⅰ Normal inspection Single sample inspection | 1.5 |
| Dimension measurement | MIL-STD-105D Inspection level Ⅰ Normal inspection Single sample inspection | 1.0 |

8-2.	INSPECTION CONDITIONS

8-2-1. THE ENVIRONMENTAL

-Room temperature: 25±3 oC

-Humidity: 65±20%RH

8-3. INSPECTION STANDARDS

8-3-1. VISUAL WHILE OPERATING

| Items to be inspected | Inspection standard |
| --- | --- |
| . No display | . If any pattern is not active at all, they can be rejected. |
| . Irregular operating | . No irregular operating are allowed . Appeared different display, which they should be chosen in the pattern, or appeared in different position where they should be chosen. |
| .Irregular display | . Any segment doesn’t active, they can be rejected. |
| . Over current | . The total current required to activate the module should not be exceed the MAX current in specification. |
| .View angles | . Valves that don’t meet the minimum value noted in the specification. they can be rejected. |
| .Contrast | . Valves that don’t meet the minimum value noted in the specification, they can be reject. |
| .LCD operate voltage | . Meet the specification. |

8-3-2. Visual while not operating

| Module dimension | . Meet the module outline drawing, not exceed the tolerance. |
| --- | --- |
| LCD panel scratch | .Following scratches inside the effective viewing area considered as the defects when their width & length are larger than the following combinations. Number: one or more Width: 0.15 length: 5.0 two or more Width: 0.10 length: 3.0 three or more Width: 0.05 length: 2.0 When the defects exceed this, it can be rejected. |

9.RELIABILITY

Standard Specification for Reliability of General-purpose LCM

| Test Item | Test Condition | Note |
| --- | --- | --- |
| High Temperature Store | 60 oC,200hr. | 2 |
| Low Temperature Store | -10 oC,200hr | 2 |
| Humidity Store | 60 oC,90%RH,96hr | 1,2 |
| High Temperature Operation | 40 oC,typical operating conditions,200hr |  |
| Temperature Cycling | -10 oC~25 oC~60 oC~25 oC 10 min. between each step temp 50 min. at each step. 5 cycles | 1,2 |
| Mechanical Vibration | 10~55Hz sweep, 3G, ampl.=10mm(max) XYZ for 10 min, each. |  |

Note 1: Condensation of water is not permitted on the module.

Note 2: The module should be inspected after 1 hour storage in normal

conditions (15~35 oC,45~65%RH)

10. HANDLING PRECAUTION

10-1. MOUNTING METHOD

The panel of the LCD module consists of two thin glass plates with polarizes which easily get damaged since the module is fixed by utilizing fitting holes in the printed circuit board. Extreme care should be taken when handling the LCD modules.

10-2. CAUTION OF LCD HANDLING & CLEANING

When cleaning the display surface. Use soft cloth with solvent (recommended below) and wipe lightly.

-Isopropyl alcohol

-Ethyl alcohol

-Tri chlorotri fluoroethane

Do not wipe the display surface with dry or hard materials that will damage the polarizes surface.

Do not use the following solvent:

-Water

-Ketone

-Aromatics

10-3.CAUTION AGAINST STATIC CHARGE

The LCD modules use COMS LSI drivers. So we recommend that you connect any unused input terminal to Vdd or Vss, do not input any signals before power is turned on and ground your body. work/assembly table. And assembly equipment to protect against static electricity.

10-4.PACKAGING

-Modules use LCD elements, and must be treated as such avoid intense shock and falls from a height

-To prevent modules from degradation, do not operate or store them exposed directly to sunshine or high temperature/humidity.

10-5.CAUTION FOR OPERATION

-It is indispensable to drive LCM within the specified voltage limit since the higher voltage than the limit shortens LCM life.

-Response time will be extremely delayed at lower temperature than the operating temperature range and on the other hand at higher temperature LCD show dark color in them.

However those phenomena do not mean malfunction or out of order with LCD, which will come back in the specified operating temperature range.

-If the display area is pushed hard during operation. Some font will be abnormally displayed but it resumes normal condition after turning off once.

-A slight dew depositing on terminals is a cause for Electro-chemical reaction resulting in terminal open circuit.

Under the maximum operating temperature, 50%RH or less is required

10-6 STORAGE

In the case of storing for a long period of time (for instance, for years) for the purpose or replacement use. the following ways are recommended

-Storage in a polyethylene bag with the opening sealed so as not to enter fresh air outside in it, and with no desiccant.

-Placing in a dark place where neither exposure to direct sunlight nor light is, keeping temperature in the specified storage temperature range.

-Storing with no touch on polarizes surface by the anythingelse.

(it is recommended to store them as they have been contained in the inner container at the time of delivery from us.

10-7.SAFETY

-It is recommendable to crash damaged or unnecessary LCD into pieces and wash off liquid crystal by using solvents such as acetone and ethanol, which should be burned up later.

-When any liquid crystal leaked out of a damaged glass cell comes in contact with your hands, please wash it off well with soap and water.

11.PRECAUTION FOR USE

11-1.A limit sample should be provided by the both parties on an occasion when the both parties agree its necessity.

Judgement by a limit sample shall take effect after the limit sample has been established and confirmed by the both parties.

11-2.On the following occasions, the handling of problem should be decided through discussion and agreement between representative of the both parties

-When a question is arisen in this specification.

-When a new problem is arisen which is not specified in this specifications.

-When an inspection specification change or operating condition change in customer is reported to GEM-TECH, and some problem is arisen in this specification due to the change.

-When a new problem is arisen at the customer’s operating set for sample evaluation in the customer size.